OVERVIEW OF THE COMPENSATION MECHANISMS FOR HOUSEHOLD SELFCONSUMPTION IN SOME EU COUNTRIES

Source: K. Varga, M. Bartek-Lesi, A. Diallo, and B. Dézsi, ‘THE CHANGING REGULATORY LAND- SCAPE OF HOUSEHOLD SELF-CONSUMP- TION’, REKK, 2024.

Payback period for different compensation systems

Case study for an average Hungarian family house

The property is a 117 m² residential house located in a small town in the Northern Hungary region, 15 km from the county seat of Borsod-Abaúj-Zemplén County. The south-facing building has two floors and includes three bedrooms, two bathrooms, a living room, a kitchen, and an entrance hall. To supply the energy needs of a household with these characteristics, 20 solar panels with an individual capacity of 370 W are required. The nominal capacity of the system is 7.4 kW, while its actual maximum annual output is 9.5 MWh. For full utilization of the system, the ideal orientation is south-facing and the ideal tilt angle is 40°, which are provided by the solar installation in question. A family of four lives in the house. The renovation was carried out in May 2021 using a home renovation subsidy (“Otthonfelújítási program”) defined in Government Decree 518/2022 (XI.25.), and its expected lifetime is approximately 20 years. The costs developed as follows.

Solar panel system installation costs (2021)

|  | Installation costs (HUF) |  | Total costs (gross, HUF) |
| --- | --- | --- | --- |
|  | Material cost (HUF) | Labor cost (HUF) |  |
| Roof renovation, replacement, insulation | 879,626 | 571,500 | 1,451,126 |
| Updating the electric system | 115,742 | 196,850 | 312,592 |
| Installation of the solar PV system | 1,660,990 | 785,812 | 2,446,802 |
| Updating the heating system | 2,032,356 | 250,000 | 2,282,356 |
| Total |  |  | 6,492,876 |

Source: Majoros G. BA thesis, 2025

The table above shows the itemized budget, from which we can obtain the following important data:

- The total cost of materials is HUF 4,688,714, and the total labor cost is HUF 1,804,162, so the total cost of the renovation is HUF 6,492,876.

- We know that the maximum amount of the subsidy is HUF 3 million, so the total own contribution is HUF 3,492,876.

Electricity production and consumption per month (kWh)

| Month | Electricity production (kWh) | Electricity consumption (kWh) | Balance (kWh) |
| --- | --- | --- | --- |
| January | 320.4 | 1326.3 | -1005.9 |
| February | 541.8 | 1079.0 | -537.2 |
| March | 808.3 | 844.4 | -36.1 |
| April | 730.0 | 600.9 | 129.1 |
| May | 1019.6 | 240.3 | 779.3 |
| June | 1051.0 | 211.9 | 839.1 |
| July | 1051.0 | 229.2 | 821.8 |
| August | 1029.5 | 214.3 | 815.2 |
| September | 766.8 | 268.2 | 498.6 |
| October | 644.4 | 739.8 | -95.4 |
| November | 262.6 | 886.0 | -623.4 |
| December | 206.4 | 1161.4 | -955.0 |
| Total | 8431.7 | 7801.7 | 630.0 |

Source: Majoros G. BA thesis, 2025

Table 1: Residential energy prices and thresholds

| Energy Product | Consumption level | Unit price (including all taxes and levies) |  |
| --- | --- | --- | --- |
|  |  | HUF | EUR |
| Electricity | <2523 kWh/year (discount band) | 36 HUF/kWh | 0.09 EUR/kWh |
|  | >2523 kWh/year | 70.104 HUF/kWh | 0.18 EUR/kWh |
| Natural gas | <1729 m3/year* (discount band) | 102 HUF/m3 | 0.26 EUR/m³ |
|  | >1729 m3/year | 747 HUF/m3 | 1.87 EUR/m³ |

Note: *1729 m3/year=63,645 MJ/year=18,088 kWh/year

Source: own compilation based on (MVM Next Energiakereskedelmi Zrt., 2025b)

Questions:

- What are the costs and revenues regarding the different compensation systems (without solar PV system, annual net metering, monthly net metering, gross metering)?

Scenario A (no solar)

7801.7 kWh  2523 * 36 HUF + (7801.7-2523) * 70.104 HUF = 460 886 HUF

Scenario B (annual net metering)

630 kWh * 5 HUF/kWh=3150 HUF

Scenario C (monthly net metering)

| Month | Balance (kWh) | Cumulative sum | Monthly net metering |
| --- | --- | --- | --- |
| January | -1005.9 | -1005.9 | -1005.9*36=-36212.4 |
| February | -537.2 | -1543.1 | -19339.2 |
| March | -36.1 | -1579.2 | -1299.6 |
| April | 129.1 |  | 129.1*5=645.5 |
| May | 779.3 |  | 3896.5 |
| June | 839.1 |  | 4195.5 |
| July | 821.8 |  | 4109 |
| August | 815.2 |  | 4076 |
| September | 498.6 |  | 2493 |
| October | -95.4 | -1674.6 | -3434.4 |
| November | -623.4 | -2298 | -22442.4 |
| December | -955 | -3253 | 730*70.104+225*36=59275.9 |
| Total | 630 | -3253 | -4036.6 |

Note: falling out of the discount band

Scenario D (gross metering)

| Month | Electricity production (kWh) | Self-consumption (ca. 30% - without a battery) | Feed-in to the grid (ca. 70% - without a battery) | Total electricity consumption (kWh) | Electricity consumption (kWh) from the grid | Cumulative sum | Revenue (selling price is 5 HUF) | Electricity bill (buying price is 36 HUF or 70,104 HUF) | Annual balance |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| January | 320.4 | 96.12 | 224.28 | 1326.3 | 1230.18 | 1230.18 | 1121.4 | 44286.5 |  |
| February | 541.8 | 162.54 | 379.26 | 1079 | 916.46 | 2146.64 | 1896.3 | 32992.6 |  |
| March | 808.3 | 242.49 | 565.81 | 844.4 | 601.91 | 2748.55 | 2829.05 | 29360.9 |  |
| April | 730 | 219 | 511 | 600.9 | 381.9 | 3130.45 | 2555 | 26772.7 |  |
| May | 1019.6 | 305.88 | 779.3 | 240.3 |  | 3130.45 | 3896.5 | 0.0 |  |
| June | 1051 | 315.3 | 839.1 | 211.9 |  | 3130.45 | 4195.5 | 0.0 |  |
| July | 1051 | 315.3 | 821.8 | 229.2 |  | 3130.45 | 4109 | 0.0 |  |
| August | 1029.5 | 308.85 | 815.2 | 214.3 |  | 3130.45 | 4076 | 0.0 |  |
| September | 766.8 | 230.04 | 536.76 | 268.2 | 38.16 | 3168.61 | 2683.8 | 2675.2 |  |
| October | 644.4 | 193.32 | 451.08 | 739.8 | 546.48 | 3715.09 | 2255.4 | 38310.4 |  |
| November | 262.6 | 78.78 | 183.82 | 886 | 807.22 | 4522.31 | 919.1 | 56589.4 |  |
| December | 206.4 | 61.92 | 144.48 | 1161.4 | 1099.48 | 5621.79 | 722.4 | 77077.9 |  |
| Total | 8431.8 | 2529.54 | 6251.89 | 7801.7 | 5621.79 |  | 31259.45 | 308065.6 | -276806.1 |
|  | falling out of the discount band |  |  |  |  |  |  |  |  |
|  | own production may cover the total consumption |  |  |  |  |  |  |  |  |

Summary of different scenarios (HUF)

|  | A (no solar) | B (annual net metering) | C (monthly net metering) | D (gross metering) |
| --- | --- | --- | --- | --- |
| Installation costs | 6492876 |  |  |  |
| Annual balance | -460886 | 3150 | -4037 | -276806 |
| Inflation | 4% |  |  |  |

How does the payback period change regarding the different compensation systems?

Payback period by annual net metering

Payback period by monthly net metering

Payback period by gross metering